DC Brushless Water Pump

| - Product Parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model Character |  |  | HM651205-4 ( Three phase pump) |  |  |  |  |  |  |
| Working voltage range |  |  | 9VDC-13.5VDC |  |  |  |  |  |  |
| Rated voltage |  |  | 12VDC |  |  |  |  |  |  |
| Rated power |  |  | ≤5W |  |  |  |  |  |  |
| Max static water head |  |  | 1.6±10% Meters |  |  |  |  |  |  |
| Static flow rate |  |  | ＞550mL/min@1.2m |  |  |  |  |  |  |
| Lifespan |  |  | Continuous operation under normal working surrounding＞20000H |  |  |  |  |  |  |
| Working Environment |  |  | PH value: PH5-10(weak acid and weak base), workable both in water and on land. |  |  |  |  |  |  |
| Max noise |  |  | Under normal working condition,(1meter over the test instrument)＜30dB |  |  |  |  |  |  |
| Max working temperature |  |  | Under room temperature surroundings, fluid medium temperature≤70℃ |  |  |  |  |  |  |
| Other optional special functions |  |  | Water shortage protection, over-voltage protection, over-current protection, blocking protection, 0-5V speed control, PWM speed control, FG signal feedback, etc. |  |  |  |  |  |  |
| Application |  |  | Air conditioner drainage system, portable air conditioner drainage system, small electrical appliance drainage system, potted plant decoration, hydroponics, humidifier, small fountain, aquarium, small medical equipment, handicrafts, etc. |  |  |  |  |  |  |
| Color |  |  | Black (Can be customized according to customer’s requirement) |  |  |  |  |  |  |
| Outlet direction |  |  | Can be adjustable at any angle |  |  |  |  |  |  |
| Dimension |  |  |  | IP grade | Inlet |  | Outlet |  | Remarks |
| Length （mm） | Width（mm） | Height（mm） |  |  | OD（mm） | ID（mm） | OD（mm） | ID（mm） | Outlet has anti-slip strip. |
| 98.0 | 82.0 | 76.0 |  | IP67 | 20.0 | 10.5 | 16.0 | 9.0 |  |
| - Outside View |  |  |  |  |  |  |  |  |  |
| - Curve Graph |  |  |  |  |  |  |  |  |  |
| Remarks: The graph is just for your reference, the final one comes with the customer required specifications. |  |  |  |  |  |  |  |  |  |
| - Photo for your reference |  |  |  |  |  |  |  |  |  |