DC Brushless Water Pump

| - Product Parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model Character |  |  | HM5010B-1( Three phase pump) |  |  |  |  |  |  |
| Working voltage range |  |  | 12VDC-24VDC |  |  |  |  |  |  |
| Rated power |  |  | 15~60 W |  |  |  |  |  |  |
| Static water head |  |  | 2~7 meters |  |  |  |  |  |  |
| Static flow rate |  |  | 10~ 22 L/min |  |  |  |  |  |  |
| Lifespan |  |  | Continuous operation under normal working surrounding＞20000H |  |  |  |  |  |  |
| Working Environment |  |  | PH value: PH5-10(weak acid and weak base), workable on land only. |  |  |  |  |  |  |
| Max noise |  |  | Under normal working condition,(1meter over the test instrument)＜45dB |  |  |  |  |  |  |
| Max working temperature |  |  | Under room temperature surroundings, fluid medium temperature≤90℃ (Aluminum shell pump body) |  |  |  |  |  |  |
| Other optional special functions |  |  | Water shortage protection, over-voltage protection, over-current protection, blocking protection, 0-5V speed control, PWM speed control, FG signal feedback, etc. |  |  |  |  |  |  |
| Application |  |  | (EV)Water circulation cooling systems, motorcycle engines, automotive water coolants, solar water circulation systems, solar water heating systems, solar pool heating systems, water heater pressure boosters, high-rise building water pressure boosters, sewage systems, lathe washing systems, car wash systems, household appliance water circulation systems, agriculture irrigation, fountain, aquarium, water tank, fish pond, crafts, etc. |  |  |  |  |  |  |
| Color |  |  | Black (Can be customized according to customer’s requirement) |  |  |  |  |  |  |
| Outlet direction |  |  | Can be adjustable at any angle |  |  |  |  |  |  |
| Dimension |  |  |  | IP grade | Inlet |  | Outlet |  | Remarks |
| Length （mm） | Width（mm） | Height（mm） |  |  | OD（mm） | ID（mm） | OD（mm） | ID（mm） | Inlet and outlet have threads. |
| 117.0 | 57.0 | 118.0 |  | IP67 | G1/2 threads | 13.0 | G1/2 threads | 13.0 |  |
| - Outside View |  |  |  |  |  |  |  |  |  |
| - Curve Graph |  |  |  |  |  |  |  |  |  |
| Remarks: The graph is just for your reference, the final one comes with the customer required specifications. |  |  |  |  |  |  |  |  |  |
| - Photo for your reference |  |  |  |  |  |  |  |  |  |