DC Brushless Water Pump

| - Product Parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model Character |  |  | HM5026-1 ( Three phase pump) |  |  |  |  |  |  |
| Working voltage range |  |  | 6VDC-24VDC |  |  |  |  |  |  |
| Rated power |  |  | 3~18 W |  |  |  |  |  |  |
| Static water head range |  |  | 0.5~5 meters |  |  |  |  |  |  |
| Static flow rate range |  |  | 2~ 8L/min |  |  |  |  |  |  |
| Lifespan |  |  | Continuous operation under normal working surrounding＞20000H |  |  |  |  |  |  |
| Working Environment |  |  | PH value: PH5-10(weak acid and weak base), workable both on land and in water. |  |  |  |  |  |  |
| Max noise |  |  | Under normal working condition,(1meter over the test instrument)＜30dB |  |  |  |  |  |  |
| Other optional special functions |  |  | Water shortage protection, over-voltage protection, over-current protection, blocking protection, 0-5V speed control, PWM speed control, FG signal feedback, etc. |  |  |  |  |  |  |
| Max working temperature |  |  | Under room temperature surroundings, fluid medium temperature≤80℃ |  |  |  |  |  |  |
| Application |  |  | Warm/cold bed mattress, cushion, solar/Gas/Electric water heater, bathtub, bathroom Kitchen equipment, medical equipment, beauty equipment, mobile bath machine, mobile air conditioner, solar fountain, aquarium, water tank, fish pond, solar water circulation, water circulation of home appliance, crafts, etc. |  |  |  |  |  |  |
| Color |  |  | Black (Can be customized according to customer`s requirement) |  |  |  |  |  |  |
| Outlet direction |  |  | Can be adjustable at any angle |  |  |  |  |  |  |
| Dimension |  |  |  | IP grade | Inlet |  | Outlet |  | Remarks |
| Length （mm） | Width（mm） | Height（mm） |  |  | OD（mm） | ID（mm） | OD（mm） | ID（mm） | Inlet and outlet have anti-slip strip. |
| 68.5 | 52.0 | 84.0 |  | IP68 | 16.0 | 12.0 | 12.0 | 7.0 |  |
| - Outside View |  |  |  |  |  |  |  |  |  |
| - Curve Graph |  |  |  |  |  |  |  |  |  |
| Remarks: The graph is just for your reference, the final one comes with the customer required specifications. |  |  |  |  |  |  |  |  |  |
| - Photo for your reference |  |  |  |  |  |  |  |  |  |